Методы решения трехмерных нестационарных уравнений Навье-Стокса в различных постановках.

Численно решается нестационарная трехмерная система уравнений Навье-Стокса, описывающая пространственное течение вязкой однородной несжимаемой жидкости, записанная как в естественных переменных, так и в переменных "вихрь-векторный потенциал". Для решения уравнений Пуассона на каждом шаге по времени используется метод неполной аппроксимации минимальных невязок с групповой оптимизацией параметров. Приводятся результаты решения некоторых тестовых задач, полученные с применением различных формулировок исходных дифференциальных уравнений.

Solving methods for the time-dependent three-dimensional Navier-Stokes equations using various formulations.

The unsteady-state incompressible Navier-Stokes equations in three-dimensions are solved numerically using both original and "rotation-vector potential" formulations. The incomplete approximation method with a multiparametric optimization is used for the Poisson's equation solution. Test cases are presented.