О НОВЫХ ОБРАТНЫХ ЗАДАЧАХ ДЛЯ ПСЕВДОПАРАБОЛИЧЕСКИХ УРАВНЕНИЙ ФИЛЬТРАЦИИ В ТРЕЩИНОВАТЫХ СРЕДАХ

А. Ш. Любанова

Сибирский федеральный университет

ON NEW INVERSE PROBLEMS FOR THE PSEUDOPARABOLIC EQUATIONS

OF FILTRATION IN FISSURED MEDIA

A.Sh.Lyubanova

Siberian Federal University, Krasnoyarsk

The investigation is devoted to the new inverse problems concerning the identification of coefficients in the second and third order terms of linear pseudoparabolic equation of filtration in a fissured rock. The coefficients depend on the permeability and hydraulic properties of the fissured rock, the intensity of the liquid transfer between the blocks and fissures. The physical and mathematical justification of possible statements of the inverse problems for pseudoparabolic equations is given. New kinds of the boundary conditions of overdetermination are discussed. Certain elliptic and parabolic inverse problems relevant to pseudoparabolic ones are considered.

Introduction

In 1960, Barenblatt, Zheltov and Kochina [1] proposed the basic concept in the theory of seepage (filtration) of homogeneous liquids in fissured rocks. A fissured rock is considered as a material consisting of pores and permeable blocks which are generally separated from each other by a system of fissures. Compared to the standard arguments of filtration in a porous medium the significant feature given in [1] lies in the fact that 1) two liquid pressures, both in the pores and in the fissures, are introduced at any point in a space and 2) the transfer of liquids between the fissures and the pores is taken into consideration. Under such an approach the model of the seepage of a liquid in a fissured rock is described by the system of equations [1]

νΔu1+αu2−u1=0, μm0d1+d2 u2t+αu2−u1=0                                          (1.1)

where  u1=u1(t,x), u2= u2(t,x) are the pressures of the liquid in the fissures and pores, respectively;  is the Laplacian operator; d1 and d2 are the coefficients of compressibility of the liquid and the blocks; m0 is the magnitude of the porosity of the blocks at standard pressure;  is the viscosity of the liquid; v represents the permeability of fissures. The dimensionless coefficient a characterizes the intensity of the liquid transfer between the blocks and fissures. In general the model can include the nonlinearities arising from fluid type (liquid or gas), concentration (porosity, absorption or saturation) and the exchange rate [2].

Eliminating u2 from (1.1) we obtain for the pressure of the liquid in the fissures u1 the so-called fissured medium equation of pseudoparabolic type

u1t−ηΔu1t−κΔu1=0 κ=νμm0d1+d2, η=να.                        (1.2)

The parameter  corresponds to the piezo-conductivity of fissured rock. The coefficient  represents a specific characteristic of fissured rock. The pressure of the liquid in the pores u2 satisfies an analogous equation. Since the natural stratum is involved, the parameters in (1.2) should be determined on the basis of the investigation of their behaviour under the natural nonsteady-state conditions but not the tests carried out on rock speciments brought to the surface. This leads to the interest in studying the inverse problems for (1.2) and its analogue.

Pseudoparabolic equations of the form

ut+L1ut+L2u=0                                                 (1.3)

with various diﬀerential operators L1 and L2 of the even order in spacial variables are also arised in the mathematical models of the heat conduction [3], wave processes [4], quasistationary processes in semiconductors and magnetics [5] (for more details, see [5] and references therein). Similar equations appears in the models for ﬁltration of the two-phase ﬂow in porous media with the dynamic capillary pressure [6].

The investigation of inverse problems for pseudoparabolic equations goes back into 1980s. The ﬁrst result obtained by Rundell in [7] is concerned with the inverse problems of the identiﬁcation of an unknown source f in the (1.3) with linear operators L1 and L2, L1 = L2. Rundell proved the global existence and uniqueness theorems in the case that f depends only on x or t. As for the determination of unknown coeﬃcients in (1.3) we mention the result of Mamayusupov [8]. He proved the uniqueness theorem and found an algorithm for solving the inverse problem with respect to u(t, x), functions b(y), c(y) and a constant a for the equation

ut−Δut=aΔu+byuy+cy+δt,x,y      for  x,y∈R2, t>0

provided that u(t, x, 0), uy(t, x, 0) and u(0, x, y) are given. Here δ(t, x, y) is the Dirac delta function. To the present author’s knowledge, inverse problems of the identiﬁcation of unknown variable coeﬃcients in the terms of the second and third order of (1.3) have not been posed and studied yet.

This work is concerned with the inverse problems of ﬁnding the unknown coeﬃcients k = k(t) or η = (t) in equation

ut−ηΔut−kΔu=f                                                       (1.4)

where f is given. The physical and mathematical justiﬁcation of new statements of the inverse problems for (1.4) are discussed.

Inverse problems on the identification of k(t)

The physical processes modeled by (1.4) occur in bounded domains. Therefore the initial and boundary conditions must be imposed for (1.4). To ﬁnd out mathematical formulation of these we start from the model (1.1).

Let a problem be considered in a domain of the stratum Ω ⊂ R3 with boundary ∂Ω, t ∈ (0, T) and T is a positive real number. The initial data for u1, u2 are [9]

−ηΔu10,x+u10,x=u0x, u20,x=u0x, x∈Ω,

respectively. Since the ﬁrst equation of (1.1) is elliptic, the boundary conditions for u1 can take the form

u1=gt,x, t∈0,T, x∈∂Ω,                                  (2.1)

∂u1∂n=gt,x, t∈0,T, x∈∂Ω,                                  (2.2)

or

Au1+B∂u1∂n=gt,x, t∈0,T, x∈∂Ω,                              (2.3)

where n is the outward unit normal to ∂Ω. u0 is given on Ω and A, B, β are given functions on (0,T)×∂Ω. From here and the second equation (1.1) we obtain the boundary conditions for u2:

ηu2t+kt u2=ktgt,x, t∈0,T, x∈∂Ω,

∂∂nηu2t+kt u2=ktgt,x, t∈0,T, x∈∂Ω,

or

Aηu2t+kt u2+B∂∂nηu2t+kt u2=ktgt,x, t∈0,T, x∈∂Ω,

Thus, we conclude that in general the initial data for (1.4) are given as

−ηΔu 0,x+u 0,x=U0x, x∈Ω,                                                 (2.4)

where U0(x) is a known function. If k(t)  0, then among the possible types of the boundary conditions the most important can be written as the condition of the Dirichlet type

u=gt,x, t∈0,T, x∈∂Ω,                                                (2.5)

or

ηut+kt u=ktg1t,x+g2t,x, t∈0,T, x∈∂Ω,                            (2.6)

the Neumann type

∂∂nηut+kt u=ktg1t,x+g2t,x, t∈0,T, x∈∂Ω,                         (2.7)

and the general mixed type

Aηut+kt u+B∂∂nηut+kt u=ktg1t,x+g2t,x, t∈0,T, x∈∂Ω,          (2.8)

Here g1 and g2 are given functions on (0, T) × ∂Ω. In the case of u1 the Dirichlet data (2.6) with g1(t,x) = g(t,x) and g2(t,x) = ηgt(t,x) comes from (2.5). The formulae (2.7),(2.8) with the same functions g1 and g2 are deduced from the appropriate Neuman and mixed boundary conditions for u1. The boundary data for u2 are of the form (2.6)–(2.8) with g1(t,x) = g(t,x) and g2(t,x) ≡ 0. Thus, three direct initial boundary value problems can be posed for (1.4) when k(t) is known.

The inverse problem of identiﬁcation of the unknown coeﬃcient k(t) with every of the above boundary conditions is underdetermined, so that in order to recover k(t) we are enforced to impose an additional condition. The identiﬁcation of k(t) here can be based on the boundary data of the pointwise or integral type. This leads to the pointwise or integral condition of overdetermination, which is in general written as

σ1ηut+kt u+σ2∂∂nηut+kt u|x=x0=kt μ1t+ψ1t, t∈0,T,       (2.9)

or

Γ ηut+kt uω1t,s+∂∂nηut+kt uω2t,s ds=kt μ2t+ψ2t, t∈0,T.      (2.10)

Here σ1, σ2 are real numbers, ω1(t,s), ω2(t,s) are given functions, x0 ∈ ∂Ω and Γ ⊆ ∂Ω. In the case of the Dirichlet boundary problem (1.4),(2.4),(2.5) after substituting (2.6) into (2.9) and (2.10) the conditions of overdetermination take the form

∂∂nηut+kt u|x=x0+kt μ1t=ψ1t, t∈0,T,                          (2.11)

Γ∂∂nηut+kt uω2t,s ds+kt μ2t=ψ2t, t∈0,T,                      (2.12)

respectively. Here

μ1t=σ2−1σ1g1t,x0−μ1t,       ψ1t=σ2−1ψ1t−σ1g2t,x0,

μ2t=Γg1t,sω1t,s ds−μ2t,        ψ2t=ψ2t−Γg2t,sω1t,s ds,

If ω2(t,s) ≡ 1 and µ2 ≡ 0, then the integral condition means a given ﬂux of a liquid through the surface Γ, for instance, the total discharge of a liquid through the surface of the ground. Similar nonlocal conditions were applied to control problems in [10] and to elliptic inverse problem in [11].

In the case of the problems (1.4),(2.4),(2.7) and (1.4),(2.4),(2.8), as the condition of overdetermination, (2.9) or (2.10) are to be taken of the form

ηut+kt u|x=x0+kt μ3t=ψ3t, t∈0,T,

Γηut+kt uω1t,s ds+kt μ4t=ψ4t, t∈0,T,

respectively. In particular, for the problem (1.4),(2.4),(2.7)

μ3t=σ1−1σ2g1t,x0−μ1t,        ψ3t=σ1−1ψ1t−σ2g2t,x0,

μ4t=Γg1t,sω2t,s ds−μ2t,         ψ4t=ψ2t−Γg2t,sω2t,s ds,

Inverse problems on the identification of (t)

Let us consider now the inverse problem of the identification of an unknown coefficient  = (t) in the model (1.1) assuming that the other coefficients of (1.1) are given constants. In this case the pressures u1 and u2 satisfies equations

u1t−ηtΔu1t−κΔu1=0                                            (3.1)

and

u2t−ηtΔu2t−κΔu2=0,                                            (3.2)

respectively. The initial data for u1, u2 are

−η0Δu10,x+u10,x=u0x, u20,x=u0x, x∈Ω.                   (3.3)

The boundary conditions for u1 can take the form of (2.1)–(2.3) and implies the following boundary conditions for u2:

ηtu2t+k u2=kgt,x, t∈0,T, x∈∂Ω,                                 (3.4)

∂∂nηtu2t+k u2=kgt,x, t∈0,T, x∈∂Ω,                              (3.5)

or

Aηtu2t+k u2+B∂∂nηtu2t+k u2=kgt,x, t∈0,T, x∈∂Ω,             (3.6)

in wiew of (1.1)2. Thus, the direct initial boundary value problems for u1 and u2 differs from one another and should to be investigated independently.

The identiﬁcation of (t) in (3.1) and (3.2) can be based on the appropriate boundary data of the pointwise or integral type as well as in the case of the unknown k(t). This leads to the pointwise and integral conditions of overdetermination, which are written for (3.1) as

σ1u1+σ2∂u1∂n|x=x0=ψ5t, t∈0,T,                                    (3.7)

Γ u1ω1t,s+∂u1∂nω2t,s ds=ψ6t, t∈0,T.                                (3.8)

where 5 and 6 are given functions. In the case of the Dirichlet boundary problem for (3.1) after substituting (2.1) into (3.7) and (3.8) the conditions of overdetermination take the form

σ2∂u1∂n|x=x0=ψ5t−σ1gt,x0,

Γ∂u1∂nω2t,s ds=ψ6t−Γgt,sω1t,s ds.

In the case of the Neuman boundary problem substituting (2.2) into (3.7) and (3.8) gives

σ1u1|x=x0=ψ5t−σ2gt,x0,

Γu1ω1t,s ds=ψ6t−Γgt,sω2t,s ds,

respectively.

The identiﬁcation of (t) in (3.2) can be based on the following pointwise or

σ1ηtu2t+k u2+σ2∂∂nηtu2t+k u2|x=x0=ψ7t, t∈0,T,

or integral condition of overdetermination

Γηtu2t+k u2ω1t,s+∂∂nηtu2t+k u2ω2t,s ds=ψ8t, t∈0,T.

In the case of the Dirichlet problem (3.2)–(3.4) substituting (3.4) into these formulae leads to the conditions

σ2∂∂nηtu2t+k u2|x=x0=ψ7t−σ1gt,x0, t∈0,T,

Γ∂∂nηtu2t+k u2ω2t,s ds=ψ8t−Γgt,sω1t,s ds, t∈0,T.

In a similar manner, one can construct the appropriate conditions of overdetermination for the problems (3.2),(3.3),(3.5) and (3.2),(3.3),(3.6).

Summaries

We discussed certain new statements of the inverse problems for pseudoparabolic equations by the example of the linear ﬁssured rock equation. The conditions of overdetermination similar to those considered above are available for the linear pseudoparabolic equations (1.3) with the operators L1 and L2 of more general form and also for the nonlinear equations

ut+L1γ1uut+L2γ2u=f

with various functions γ1 and γ2 arising in generalized models of the liquid ﬂow in porous media [2],[12].

References

- [1] Barenblatt G.I., Zheltov Iu.P., Kochina I.N. Basic concepts in the theory of seepage of homogeneous liquids in ﬁssured rocks [strata]. J. Appl. Math. Mech. Vol.24. 1960. P. 1286–1303.

- [2] B¨ohm M. Showalter R.E. Diﬀusion in ﬁssured media. SIAM J. Math. Anal. Vol.16. 1985. P. 500-509.

- [3] Chen P.J., Gurtin M.E. On a theory of heat conduction involving two temperatures. Z.

Angew. Math. Phys. Vol.19. 1968. P. 614–627.

- [4] Benjamin T.B., Bona J.L., Mahony J.J. Model equations for long waves in nonlinear dispersive systems, Philos. Tranc. Royal Soc. London. Ser. A. Vol.272. 1972. P. 47–78.

- [5] Korpusov M.O., Sveshnikov A.G. Blow-up of solutions of strongly nonlinear equations of pseudoparabolic type. J. Math. Sciences. Vol.148. 2008. P. 1–142.

- [6] Helmig R., Weiss A. Wohlmuth B.I. Dynamic capillary eﬀects in heterogeneous porous media. Comput. Geosciences. Vol.11. 2007. P. 261-274.

- [7] Rundell W. Determination of an unknown nonhomogeneous term in a linear partial diﬀerential equation from overspeciﬁed boundary data, Appl. Anal. V.10. 1980. P. 231–242.

- [8] Mamayusupov M.Sh. The problem of determining coeﬃcients of a pseudoparabolic equation, Studies in integro-diﬀerential equations. Frunze:Ilim. 1983. No. 16. P. 290–297.

- [9] Barenblatt G.I., On certain initial boundary value problems for the equations of seepage of a liquid in ﬁssured rocks. J. Appl. Math. Mech. Vol.2. 1963. P. 348–350.

- [10] Li T.-T., White L.W. Total Flux (Nonlocal) Boundary Value Problems for Pseudoparabolic Equation. Appl. Anal. Vol.16. 1983. P. 17–31.

- [11] Lyubanova A.Sh. Identiﬁcation of a constant coeﬃcient in an elliptic equation. Appl. Anal. Vol.87. 2008. P. 1121–1128.

- [12] Barenblatt G.I., Garcia-Azorero J., De Pablo A., Vazquez J.L. Mathematical Model of the Non-Equilibrium Water-Oil Displacement in Porous Strata. Appl. Anal. Vol.65. 1997. P. 19–45.